Release Note

Firmware Version 1.0.0.48

Feb. 22, 2007

# Release Notes of 1.0.0.48. Changes since last build of BETA2 release 1.0.0.40

- Support configuration of Enable Disconnet Tone Detection

- Add configuration support of min delay before dialing after offhook

- Add configuration support of PSTN CID relay to use SIP From or SIP P-Asserted-Identity.

- remove duplicated silence suppression under ADV

- enhance Status Page showing more info if line is busy

- support configurable item of PSTN current disconnect threshold under FXO/Lines

- support space character in dialplan grammar input.

- Fix: pstn incoming call gets rings forever if sip profile is set to SIP Register NO

- Fix software reboot doesn’t work after a pstn to voip call if 1 stage dialing w/o offhook auto dial is configured

- fix port hang after a previous offhook auto dialed INV failed because server is down  and 1 stage dialing is configured without offhook auto dial.

- Support dial plan for voip to pstn 1 stage dialing (see notes and web UI for grammer)

- Corrected type error of "FXO Linnes" if Status Page is active in previous build

- Removed extra "Inte-digit Timeout" under ADVANCED (duplicate with this under FXO Lines tab.

- Add support of T38 ECM mode and configuration

- Support No STUN message but send keep alive packet "No, but send keep-alive"

- Corrected. telnet interface shows GXV-3000 as command shell prompt, instead of GXW-4100

- Support FXO voip to PSTN call supervision via polarity reversal service on the PSTN line(web UI under FXO/Lines, see following notes for usage

- Support Global Touch style of call transfer, using 'Refer-To Uses Target Contact' for call transfer, default to NO(GlobalTouch style server set to YES)

- Support configurable Silence Suppression and Voice Frames per TX UI (Advanced Settings)

- Improved T38 success transmission rate for concurrent faxing

- Added support for challenging Broadsoft remote-reboot NOTIFY (replies "401 Unauthorized" with WWW-Authenticate header).

- Fixed we did not use the same Authorization credential in ACK as in INVITE

- Fixed we do not follow Retry-After as indicated in 500/503 for REGISTER

- Fixed we respond to incoming non-INVITE requests with incorrect account when talking on a different account

- Added under Broadsoft mode, register delay after 403 changed to 20 minutes (otherwise 60 minutes)

- Support of DHCP option 61 (client identifier) and removed DHCP option 57   (maximum DHCP message size)

- Added option to disable call-waiting tone